# SECTION 27 21 29

DATA COMMUNICATIONS SWITCHES AND HUBS

Equipment Specified in this section:

Crestron CEN-SWPOE-16

Table of Contents

PART 1	GENERAL	3

1.1	SUMMARY	3

A.	Section Includes:	3

B.	Related Requirements:	3

1.2	REFERENCES	4

A.	Abbreviations and Acronyms	4

B.	Reference Standards	4

PART 2	PRODUCTS	4

2.1	NETWORK SWITCH	4

A.	Manufacturers:	4

B.	Basis of Design Product:	4

C.	Minimum Functional Requirements:	4

D.	PoE	5

E.	Control Processor Accessible PoE Parameters	6

F.	Control Processor Accessible PoE Status Feedback	6

G.	Connectors	6

H.	Front Panel Indicators	6

I.	Rear Panel Indicators	7

J.	Power Requirements	7

K.	Enclosure	7

L.	Environmental	8

PART 3	EXECUTION	8

END OF SECTION 27 21 29	8

# SECTION 27 21 29

DATA COMMUNICATIONS SWITCHES AND HUBS

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SUMMARY

Section Includes:

Managed Ethernet switch with 16 PoE ports.

Specifier: Related requirements paragraph is optional.  If retaining, edit and coordinate list of sections below to correspond to Project requirements.

Related Requirements:

Section 12 24 13 — Roller Window Shades

Section 23 09 23 — Direct Digital Control System for HVAC

Section 25 08 00 — Commissioning of Integrated Automation

Section 25 10 00 — Integrated Automation Network Equipment

Section 25 11 13 — Integrated Automation Network Servers

Section 25 13 13 — Integrated Automation Control and Monitoring Network Supervisory Control

Section 25 13 16 — Integrated Automation Control and Monitoring Network Integration Panels

Section 25 13 19 — Integrated Automation Control and Monitoring Network Interoperability

Section 25 15 16 — Integrated Automation Software for Control and Monitoring Networks

Section 25 50 00 — Integrated Automation Facility Controls

Section 26 09 43.13 — Digital-Network Lighting Controls

Section 26 05 00 — Common Work Results for Electrical

Section 26 09 23 — Lighting Control Devices

Section 26 09 36 — Modular Dimming Controls

Section 26 09 43 — Network Lighting Controls

Section 27 15 00 — Communications Horizontal Cabling

Section 27 41 00 — Audio-Video Systems

REFERENCES

Abbreviations and Acronyms

PoE: Power over Ethernet.

DTE: Data Terminal Equipment.

MDI: Media Dependent Interface.

PSE: Power Sourcing Equipment.

PD: Powered Device.

BTU: British Thermal Unit

Reference Standards

Specifier: Edit below paragraphs to include only those standards that are referenced in this section.

Underwriters Laboratories, Inc. (UL):

UL Standards and requirements listed.

Federal Communications Commission (FCC):

FCC Part 15, Class B digital device compliance.

Industry Canada (IC):

CAN ICES-3(B)/NMB-3(B) - Information Technology Equipment (ITE)

Institute of Electrical and Electronic Engineers (IEEE):

IEEE Std 802.3 - Ethernet based LAN, (Ethernet).

IEEE Std 802.3u - 100 Mb/s (Fast Ethernet), Type 100base-T.

IEEE Std 802.3ab - 1000 Mb/s Operation over copper, Type 1000Base-T.

IEEE Std 802.3af - Data Terminal Equipment (DTE) Power via Media Dependent Interface (MDI), Power-over-Ethernet (PoE).

IEEE Std 802.3at – Power over Ethernet (PoE+)

PRODUCTS

NETWORK SWITCH

Manufacturers:

Basis-of-Design Manufacturer:  Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767-1903, www.crestron.com [or comparable products from a single manufacturer approved by Architect prior to bidding], with the following components and characteristics.

Basis of Design Product:

Crestron CEN-SWPOE-16

Minimum Functional Requirements:

Power over Ethernet

Switchable PoE on all ports

Switches without individual control of port PoE power shall not be accepted.

Port speed configuration:

Individually configurable per port

Detect and Auto-Negotiate

Duplex Mode:

Individually configurable per port

Detect and Auto-Negotiate

Auto MDI and MDIX

Switches requiring separate crossover Ethernet cables shall not be accepted.

Layer 2 managed switching

Port Mirroring

Multiple Configuration methods:

Web browser interface

Software application via direct USB connection

Front panel port label

PoE

General:

PoE enabled ports: 16

Power:

Switch shall meet or exceed the following:

Internal PoE power supply: 255 watts (total for all ports combined)

IEEE 802.3at Type 1, Class 0-3

Maximum power per port: 15.4 watts

Supported by all ports simultaneously.

IEEE 802.3at Type 2, Class 4

Maximum power per port: 34.2 watts

Supported by all ports up to the maximum power capability of the main PoE power supply.

PoE Parameters:

PoE enabled ports shall individually support the following:

Force On mode: PoE power is forced to ON for use with PDs that do not meet IEEE PoE requirements.

Enable: Enable PoE power to selected port.

Enable on Boot:  Enable PoE power to selected port on switch boot.

Allocated Power: a predetermined PoE power level in watts is allocated from the overall available PoE power supply total.

Control Processor Accessible PoE Parameters

The following parameters shall be available to remote control processor.

PoE power: Enable/Disable

PoE ports shall be capable of individual PoE power control via remote control processor.

Switch shall provide status feedback.

PoE state at switch boot up

Control Processor Accessible PoE Status Feedback

Link Status: Good/Down

Data Transmission: Full/Half Duplex

Link Speed

PoE device connected

Port PoE power usage in milliwatts

Port PoE power allocated in milliwatts

Port Poe power class

Connectors

Ethernet 10/100/1000Base-T ports:

Number of built-in ports: 16

Connector:

Female RJ-45

Power:

Number of connectors: 1

Connector Type:

IEC C14 Appliance Inlet

Chassis Ground:

Number of connectors: 1

Connector Type:

Single 6-32 screw

USB

Number of connectors: 1

Connector:

USB Type-B female

Front Panel Indicators

The following LED indicators shall be provided on the front panel of switch chassis:

Device Power Status

Port Speed and Status, one indicator for each port

Port Activity, one indicator for each port

Port PoE Status, one indicator for each port

Rear Panel Indicators

The following LED indicators shall be integrated into each RJ-45 port:

Port Speed and Status

Ethernet port activity

Power Requirements

Internal auto-sensing power supply:

Voltage:

Minimum: 100 VAC

Maximum: 240 VAC

Frequency:

Minimum: 50 Hertz

Maximum: 60 Hertz

Current:

Minimum Current: 1.6 Amps

Maximum Current: 3.5 Amps

Enclosure

Dimensions:

Height:

1.73 inches (44 mm)

Width:

17.28 inches (439 mm) rack mounting ears removed

19.0 inches (843 mm) rack mounting ears attached

Depth:

10.06 inches (256 mm)

Weight:

6.4 pounds (2.9 kg)

Mounting

Standard 19 inch rack mountable, 1 rack units high.

Rack mounting ears shall be removable for free standing applications.

Ventilation

Fan cooled

Vented side panels

Environmental

Operating Temperature:

32-104 degrees Fahrenheit (0-40 degrees Celsius)

Humidity:

10-90 percent Relative Humidity (non-condensing)

Heat Dissipation:

171 BTU/hour

EXECUTION

Not Used

END OF SECTION 27 21 29