SECTION 27 41 16.70

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Equipment Specified in this section:

HD-XSPA

Table of Contents

SECTION 27 41 16	3

PART 1	GENERAL	3

1.1	SECTION INCLUDES	3

A.	High Definition Surround Processor System	3

PART 2	PRODUCTS	3

2.1	MANUFACTURERS	3

A.	Basis-of-Design Manufacturer:	3

2.2	4K HDMI SURROUND SOUND PROCESSOR	3

A.	Basis of Design Product:	3

B.	System Architecture	3

C.	Front Panel Control Interface	4

2.3	SURROUND SOUND AUDIO	4

A.	Surround Sound Processor Decoding Modes:	4

B.	Analog audio output	4

2.4	AUDIO AMPLIFIER	4

A.	Integrated Audio Power Amplification	4

2.5	VIDEO MATRIX SWITCHER	4

A.	Switcher Signal Routing	4

B.	HDMI inputs:	5

C.	Transmitter receive input:	5

D.	HDMI Outputs:	6

2.6	SYSTEM CONTROL INTERFACE	6

A.	Control Master	6

B.	Local Control Port Minimum Characteristics:	6

PART 3	EXECUTION - NOT USED	8

END OF SECTION 27 41 16	8

# SECTION 27 41 16

INTEGRATED AUDIO-VIDEO SYSTEMS AND EQUIPMENT

Specifier: The Specifier/Design Professional is responsible for the accuracy of all project specifications, including system application and coordination with related sections.  This guide specification is provided as a convenience and requires editing to match actual project requirements.  CRESTRON ELECTRONICS, INC. SHALL NOT BE LIABLE FOR ANY DAMAGES ARISING OUT OF THE USE OF ANY OF ITS GUIDE SPECIFICATIONS.  For Crestron design assistance and design review please contact Sales Support Services Department at   800.237.2041 or techsales@crestron.com.

GENERAL

SECTION INCLUDES

High Definition Surround Processor System

The HD switching Surround Sound Processor and control interface unit integrates 4K HDMI transmission receive capability, 4K HDMI switching, audio switching, surround sound processing and amplification, and local device control.

PRODUCTS

MANUFACTURERS

Basis-of-Design Manufacturer:

Subject to compliance with requirements, provide products of Crestron Electronics, Inc., Rockleigh, NJ 07647, Phone 800-237-2041, Fax: 201-767-1903, www.crestron.com [or comparable products from a single manufacturer approved by A/E prior to bidding], with the following components and characteristics.

4K HDMI SURROUND SOUND PROCESSOR

Basis of Design Product:

Crestron, HD-XSPA High Definition Surround Sound AV Receiver.

System Architecture

The Surround Sound Processor shall be a single integrated Surround Sound Processor, switcher, amplifier and control interface unit integrating the following functions:

4K HDMI switching.

HDBaseT and 4K HDMI transmission receiver

Analog and digital audio switching

Stereo mix-down output

8 channel audio amplifier

Line level and amplified sub-woofer output

Device control interface

The Surround Sound Processor shall integrate the following external devices utilizing a control processor and user interface by same manufacturer:

External Single Cable HDBaseT or 4K Signal transmitter or transmitting matrix switcher

4K HDMI source devices

4K HDMI Video display

Local system devices under control

Front Panel Control Interface

The front panel user interface shall provide the following system functions:

System Setup

Basic control

SURROUND SOUND AUDIO

Surround Sound Processor Decoding Modes:

None, Stereo, Dolby Pro Logic IIx Movie, Dolby Pro Logic IIx Music, DTS Neo:6 Cinema, DTS Neo:6 Music, Two Channel Steering – Surround, Two Channel Steering – Rear, Multi-Channel Stereo (Party), Dolby Digital, Dolby Digital EX, Dolby Digital Plus, Dolby TrueHD, DTS, DTS-ES Matrix, DTS-ES Discrete, DTS 96/24, DTS-HD Master Audio™, PCM Multi-Channel

Analog audio output

Pre-amp Down-mix:  2 female RCA connectors

Pre-amp sub-woofer: 1 female RCA connector

AUDIO AMPLIFIER

Integrated Audio Power Amplification

8 amplifier channels with the following options:

7.1 Surround – 140 Watts per channel at 8 ohms, 280 Watts per channel at 4 ohms

5.1 Surround plus Zone-2 stereo down-mix

Connectors:

(8) 2-pin 7.62mm 15A detachable terminal blocks

VIDEO MATRIX SWITCHER

Switcher Signal Routing

Any video input signal shall be routable to the 4K HDMI output.

The internal 4K video switcher shall have multiple inputs.

Four 19-pin Type A HDMI female connector

One 8-pin RJ-45 female connector with support for:

4K Single Cable Signal Transmission protocol

HDBaseT compliant signals

Video matrix switching shall be executed in the digital domain.

HDMI inputs:

Connectors:  four (4) 19-pin type A female HDMI

Format:

4K HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, 2560x1440@60Hz, 2560x1600@60Hz, 3840x2160@24Hz, 3840x2160@30Hz, 4096x2160@24Hz, plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Transmitter receive input:

Connector: 1 female 8-pin RJ-45 connector

Format:

4K HDMI with Deep Color and 3D

HDCP content protection

Input video resolutions:

Progressive video resolutions:

640x480@60Hz, 720x480@60Hz (480p), 720x576@50Hz (576p), 800x600@60Hz, 848x480@60Hz, 852x480@60Hz, 854x480@60Hz, 1024x768@60Hz, 1024x852@60Hz, 1024x1024@60Hz, 1280x720@50Hz (720p50), 1280x720@60Hz (720p60), 1280x768@60Hz, 1280x800@60Hz, 1280x960@60Hz, 1280x1024@60Hz, 1360x768@60Hz, 1365x1024@60Hz, 1366x768@60Hz, 1400x1050@60Hz, 1440x900@60Hz, 1600x900@60Hz, 1600x1200@60Hz, 1680x1050@60Hz, 1920x1080@24Hz (1080p24), 1920x1080@25Hz (1080p25), 1920x1080@50Hz (1080p50), 1920x1080@60Hz (1080p60), 1920x1200@60Hz, 2048x1080@24Hz, 2048x1152@60Hz, 2560x1440@60Hz, 2560x1600@60Hz, 3840x2160@24Hz, 3840x2160@30Hz, 4096x2160@24Hz, plus any other resolution allowed by HDMI up to 300MHz pixel clock.

Interlaced video resolutions:

720x480@30Hz (480i), 720x576@25Hz (576i), 1920x1080@25Hz (1080i25), 1920x1080@30Hz (1080i30), plus any other resolution allowed by HDMI up to 300MHz pixel clock.

HDMI Outputs:

Connector:  One (1) 19-pin type A female HDMI

Format:

4K HDMI with Deep Color and 3D

HDCP content protection

Output video resolution: Matched to input resolution

SYSTEM CONTROL INTERFACE

Control Master

The Switching Surround Processor unit shall connect to an external control processor by same manufacturer via 10/100 BaseT Ethernet.

Local Control Port Minimum Characteristics:

Infrared Output

Four 2-pin 3.5mm detachable terminal blocks, IR/Serial output ports;

IR output up to 1.2 MHz; One-way serial TTL/RS-232 (0-5 Volts) 2 up to 9600 baud .

Digital Input

One 5-pin 3.5mm detachable terminal block.

Comprised of 4 digital/contact closure inputs.

Rated for 0-24 Volts DC, referenced to GND.

Input Impedance: 2.2k ohms pulled up to 5 Volts DC.

Logic Threshold: 2.5 Volts DC nominal with 1 Volt hysteresis band.

Relay

One 4-pin 3.5mm detachable terminal block.

Comprised of 2 normally open, isolated relays.

Rated 1 Amp, 30 Volts AC/DC.

MOV arc suppression across contacts.

Serial Communication Port

One 5-pin 3.5mm detachable terminal block, bidirectional RS-232 ports.

Up to 115.2k baud, hardware and software handshaking support for communication with serial devices.

Ethernet

One 8-wire RJ45 with 2 LED indicators.

10/100BaseT Ethernet port.

EXECUTION - NOT USED

END OF SECTION 27 41 16